- Code mô hình hóa thuật toán PID (Activity Diagram – UML):

@startuml

title Activity Diagram - PID-based Active Balancing Control Loop

start

:Initialize PID parameters\n(Kp, Ki, Kd, dt);

:Initialize state variables\n(previous_error = 0,\nintegral = 0);

repeat

:Read vibration amplitude\nfrom sensor;

:Compute error\nerror = SET_POINT - measured_amplitude;

:Proportional term\nP = Kp * error;

:Update integral\nintegral = integral + error * dt;

if (Integral > MAX_LIMIT?) then (Yes)

:integral = MAX_LIMIT;

else if (Integral < MIN_LIMIT?) then (Yes)

:integral = MIN_LIMIT;

endif

:Integral term\nI = Ki * integral;

:Derivative term\nD = Kd * (error - previous_error) / dt;

:Compute PID output\noutput = P + I + D;

if (Output > output_max?) then (Yes)

:output = output_max;

else if (Output < output_min?) then (Yes)

:output = output_min;

endif

:Update previous_error\nprevious_error = error;

:Send control command\nto servo / actuator;

:Wait for next sampling\ninterval (dt);

repeat while (System running?)

stop

@enduml

- Code mô hình hóa toàn bộ hệ thống:

@startuml

title Overall Activity Diagram - Vibration-Based Machinery Health Monitoring\nwith Active Balancing System

start

:Initialize system;

:Configure MPU6050 sensor;

:Initialize PID controller;

:Initialize communication (MQTT / HTTP);

repeat

:Collect vibration data\n(ax, ay, az);

:Preprocess acceleration signals;

:Perform FFT analysis;

:Extract vibration features\n(frequency, amplitude, rms, peak);

if (Imbalance detected?) then (Yes)

:Compute vibration error;

:PID control computation;

:Generate control signal;

:Drive actuator / DC motor;

:Adjust counterweight position;

else (No)

:Maintain current balance state;

endif

:Log vibration data\nand system status;

:Update Web dashboard;

:Transmit data via MQTT / HTTP;

repeat while (System running?)

stop

@enduml